Lab no 07:  Arithmetic and Logic Operations (ALU)

The purpose of this Lab is to learn to:

- Implement a 2-bits Arithmetic and Logic Operations (ALU) on FPGA. You will write the behavioral description for the ALU using Verilog.

- Connect sub-modules on a top-level module. You will connect the ALU and the seven-segment decoder.

Be aware: In this lab, the seven-segment decoder is written in behavioral description using a case statement, not logical equations like Lab 2.

- Use the switches and seven-segment on the FPGA board to display the output of the ALU.

Parts: -

- Code the behavioral description of the 2-bit ALU.

- Code the behavioral description of the seven-segment decoder.

- Connect the ALU and the seven-segment decoder on the top-level module and run it on FPGA.

Part 1. Code the behavioral description of the 2-bit ALU

One of the core components of a CPU is the Arithmetic and Logic Unit (ALU). This component is responsible for performing the calculations (such as addition) on data when executing a machine code instruction. It is like a simple calculator where you set the inputs and the operation, giving you the result.

In this lab, we will design and build a basic ALU that is capable of eight operations, as follows:

| ALU Selection | ALU Operation | ALU Output |
| --- | --- | --- |
| 000 | ALU_Out = A and B; | On 2 Leds |
| 001 | ALU_Out = A or B; | On 2 Leds |
| 010 | ALU_Out = A + B; | On 1 Seven Segment |
| 011 | ALU_Out = A - B; | On 1 Seven Segment |
| 100 | ALU_Out = A * B; | On 1 Seven Segment |
| 101 | ALU_Out = A > B; | Draw A on Seven Segment |
| 110 | ALU_Out = A < B; | Draw B on Seven Segment |
| 111 | ALU_Out = A == B; | Draw = on Seven Segment |

Behavioral description Verilog code for ALU

module ALU ( A, B, ALU_sel, ALU_result);

input [1:0] A,B; // ALU 2-bit Inputs

input [2:0] ALU_sel; // ALU 3-bit Selector

output [3:0] ALU_result; // ALU 4-bit Output

reg [3:0] ALU_result;

always @(*)

begin

case(ALU_sel)

3'b000: // Logical and

ALU_result = A & B;

3'b001: // Logical or

ALU_result = A | B;

3'b010: // Addition

ALU_result = A + B;

3'b011: // Subtraction

ALU_result = A - B;

3'b100: // Multiplication

ALU_result = A * B;

3'b101: // Greater comparison

ALU_result = A > B? 4'b1010:4'b1011;

3'b110: // lower comparison

ALU_result = A < B? 4'b1011:4'b1010;

3'b111: // Equal comparison

ALU_result = A == B? 4'b1100:4'b0000;

default: ALU_result = 4'b0 ;

endcase

end

endmodule

Part 2. Code the behavioral description of the seven-segment decoder

Seven segments decoder using the Case statement. A seven-segment display decoder takes a 4-bit data input BCD and produces seven outputs to control light-emitting diodes to display a digit from 0 to 9 and comparison outputs.

Behavioral description Verilog code for seven segments decoder.

module sevenSegments (bcd,dec);

input [3:0]bcd;

output reg [6:0] dec;

always@(bcd)

begin

case(bcd)

4'b0000 : dec = ~7'b1111110; // 0

4'b0001 : dec = ~7'b0110000; // 1

4'b0010 : dec = ~7'b1101101; // 2

4'b0011 : dec = ~7'b1111001; // 3

4'b0100 : dec = ~7'b0110011; // 4

4'b0101 : dec = ~7'b1011011; // 5

4'b0110 : dec = ~7'b1011111; // 6

4'b0111 : dec = ~7'b1110000; // 7

4'b1000 : dec = ~7'b1111111; // 8

4'b1001 : dec = ~7'b1111011; // 9

4'b1010 : dec = ~7'b1110111; // A

4'b1011 : dec = ~7'b1111111; // B

4'b1100 : dec = ~7'b1001000; // =

default : dec = ~7'b1111110; //0 default

endcase

end

endmodule

Part 3. Connect the ALU and the seven-segment decoder on the top-level module and run it on FPGA

module ALU_to_decoder (A, B, ALU_sel,Segmentleds,leds);

input [1:0] A,B; // ALU 2-bit Inputs

input [2:0] ALU_sel; // ALU 3-bit Selector

output [6:0] Segmentleds; //Seven segment leds

output [1:0] leds; // Leds

wire [3:0] ALU_result; // ALU 4-bit Output

// Instantiation of the  ALU

ALU ALU_dut(A, B, ALU_sel, ALU_result);

// Instantiation of the  Seven segment //Display Output on Seven segments

sevenSegments sevenSegments_dut (ALU_result,Segmentleds);

assign leds = ALU_result [1:0]; //Diplay Output on Leds

endmodule

Run the integrated design (ALU + Decoder) on FPGA.

- Refer to Lab 2 to program the FPGA by Quartus

- Use DE10-lite kit, Altera MAX 10 based FPGA board

- Check the DE10-lite user manual (Here) for pin assignment.

Note: Inputs/Outputs Assignment, Open the Assignment tab, click on Pin Planner, and assign pins as shown in the figure below.